Prepared Modules for Exam

Lab1:

-Read the state of the dip switch and send it to the LED-

START: LDI R17, $00 ;LOAD R17 = 00 LDI R16, $FF ;LOAD R16 = FF OUT DDRB, R16 ;SET DDRD = 1 --> OUTPUT OUT DDRA, R17 ;SET DDRB = 0 --> INPUT OUT PORTA, R16 ;SET PULL UP REGISTER AT PORTA READ_PIN: IN R18, PINA ;READ INPUT VALUE FROM PINA TO R18 OUT PORTB, R18 ;LOAD VALUE OF R18 TO PORTB RJMP READ_PIN ;READ AGAIN

-Delay 1ms (Timer 0)-

- Delay1ms: - /*XTAL is 8MHz, we scale down 64*/ - Delaymodule: - LDI R16,(1<<0) ;SET BIT 0 - SBI DDRA,0 - LDI R17,0 - OUT PORTA,R17 - LDI R20,124 ;STORE 124 INTO OCR0A - OUT OCR0A,R20 - Delay1ms: - LDI R20,0x02 ;SET WGM01 ON - OUT TCCR0A,R20 - LDI R20,0x03 ;SET CSO1 AND CSO2 - OUT TCCR0B,R20 - AGAIN: - SBIS TIFR0,OCF0A - RJMP AGAIN - LDI R20,0x0 - OUT TCCR0B,R20 - LDI R20,1<<OCF0A - OUT TIFR0,R20 - RET

-Delay  (Timer1)-

DELAY_1ms: LDI R20, 0x00 STS TCNT1H, R20 STS TCNT1L, R20 LDI R20, LOW(124) ;124 STS OCR1AL, R20 LDI R20, HIGH(124) STS OCR1AH, R20 LDI R20, 0x00 STS TCCR1A, R20 LDI R20, (1 << WGM12) ¦ (1 << CS11) ¦ (1 << CS10);prescale 64 STS TCCR1B, R20 AGAIN: SBIS TIFR1, OCF1A RJMP AGAIN LDI R19, 0x00 STS TCCR1B, R19 STS TCCR1A, R19 LDI R20, 1<<OCF1A OUT TIFR1, R20 RET

-Shift Register-

.INCLUDE "m324padef.inc" .DEF shiftDataReg = R22 .EQU configPort = PORTB .EQU configDDR = DDRB .EQU n_clearPin = 3 ;MCLR .EQU shiftSHCP = 2 ;CLOCK .EQU shiftLATCH = 1 ;LATCH .EQU shiftSDI = 0 ;SDI //MAIN CALL INIPORT MAIN: LDI R26, 8 LDI shiftDataReg, 0x55 ;0101 0101 CALL SHIFT_DATA STOP: JMP STOP //END MAIN //SUBROUTINES //INIPORT INIPORT: LDI R24, (1<<n_clearPin)¦(1<<shiftSHCP)¦(1<<shiftLATCH)¦(1<<shiftSDI) OUT configDDR, R24 ;Set 1 --> output SBI configPort, n_clearPin ;Set clear pin to 1 --> working RET //END INIPORT //SHIFT_DATA SHIFT_DATA: MOV R23, shiftDataReg ANDI R23, 0b1000000 CPI R23, 0x00 BRNE BIT_HIGH BREQ BIT_LOW RET //END SHIFT_DATA //BIT_HIGH BIT_HIGH: SBI configPort, shiftSDI SBI configPort, shiftSHCP CBI configPort, shiftSHCP SBI configPort, shiftLATCH CBI configPort, shiftLATCH CALL SHIFT_DELAY LSL shiftDataReg DEC R26 BREQ MAIN RJMP SHIFT_DATA //END BIT_HIGH //BIT_LOW BIT_LOW: CBI configPort, shiftSDI SBI configPort, shiftSHCP CBI configPort, shiftSHCP SBI configPort, shiftLATCH CBI configPort, shiftLATCH CALL SHIFT_DELAY LSL shiftDataReg DEC R26 BREQ MAIN RJMP SHIFT_DATA //END BIT_LOW ;delay0.5s SHIFT_DELAY: **INSERT DELAY HERE**

-LCD Display-

.EQU LCD_PORT =PORTA ; Define LCD port .EQU LCD_DDR =DDRA ; Define LCD data direction .EQU RS =0 ; Register Select pin .EQU RW =1 ; Read/Write pin .EQU EN =2 ; Enable pin .ORG 0 //INITIALIZE LDI R16, LOW(RAMEND) ; Load the low address of RAMEND into R16 OUT SPL, R16 ; Output the low byte to the stack pointer low LDI R16, HIGH(RAMEND) ; Load the high address of RAMEND into R16 OUT SPH, R16 ; Output the high byte to the stack pointer high // //MAIN MAIN: CALL PRESET // Call the subroutine to initialize the power (POWERUP_LCD_4BIT) for 4-bit LCD mode. Refer to the curriculum for specifics. Do not call this subroutine; otherwise, the LCD will not display anything. CALL POWERUP_LCD_4BIT ;Use this subroutine to turn on LCD // Call the subroutine to configure the LCD. CALL INIT_LCD_4BIT ;Configure the LCD // Call the command to set the DDRAM pointer to 0x00. LDI R17, 0x00 ; CALL WRITE_COMMAND ;Write command to LCD // Below is the code to reference the table and display characters from the first row on the LCD until a 0x00 character is encountered. LDI ZH, HIGH(LINE1 << 1) ; Load high byte of LINE1 address into ZH LDI ZL, LOW(LINE1 << 1) ; Load low byte of LINE1 address into ZL DISPLAY_LINE1: LPM R17, Z+ ; Load character from the program memory into R17 and then count up CPI R17, $00 ; Compare R17 with 0x00 BREQ NEWLINE ; Branch to NEWLINE if equal CALL WRITE_DATA ; Call to write data to LCD RJMP DISPLAY_LINE1 ; Repeat the process for the next character NEWLINE: // Write command to move to the next line (command to set DDRAM address at 0x40) LDI R17, 0xC0 CALL WRITE_COMMAND // Similarly, write code to display on the second row //Display line 2 DISPLAY_LINE2: LPM R17, Z+ ; Load character from the program memory into R17 CPI R17, $00 ; Compare R17 with 0x00 BREQ LOOP ; Branch to NEWLINE if equal CALL WRITE_DATA ; Call to write data to LCD RJMP DISPLAY_LINE2 ; Repeat the process for the next character //END MAIN //LOOP LOOP: // This part of the code loops infinitely without any actions, as everything only needs to run once. RJMP LOOP //END LOOP // Subroutine ;------------------------------------------------ // PRESET // Input: None // Output: None // Description: Preset for LCD before further execution PRESET: LDI R20, $FF ;R20 = 1111 1111 OUT LCD_DDR, R20 ;Load direction = 1 --> DDRA is in output state LDI R20, $00 ;R20 = 0000 0000 OUT LCD_PORT, R20 ;Clear all values of PORTA SBI LCD_PORT, RW ;Set bit to READ CBI LCD_PORT, RS ;Clear bit Register Select CBI LCD_PORT, EN ;Clear bit Enable RET // POWERUP_LCD_4BIT // Input: None // Output: None // Description: Subroutine to power up the LCD POWERUP_LCD_4BIT: //Wait for the LCD to turn on --> need delay LDI R16, 250 ; Load 250 into R16 RCALL DELAY_US ; Call the delay subroutine //Command for 8 bit LDI R17, $28 ; Load command to initialize LCD in 8-bit mode RCALL OUT_COMMAND ; Call to output command //Delay LDI R16, 20 ; Load 50 into R16 RCALL DELAY_US ; Call the delay subroutine // RET // LCD Initialization Subroutine // Input: None // Output: None // Description: Initializes the LCD INIT_LCD_4BIT: LDI R17, $01 ; Command to clear the display CALL WRITE_COMMAND ; Call to write command LDI R17, $28 ; Command for 4-bit mode, 2 lines, 5x8 dots CALL WRITE_COMMAND ; Call to write command LDI R17, $0C ; Command to turn on display, cursor off CALL WRITE_COMMAND ; Call to write command //LDI R17, $06 ; Command to shift cursor right //CALL WRITE_COMMAND ; Call to write command RET // Subroutine to write command to LCD // Input: R17 // Output: None // Description: Writes command contained in R17 to the LCD, this goes with OUT_COMMAND WRITE_COMMAND: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output the first 4 bits RCALL OUT_COMMAND ; Call to output command POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output the last 4 bits RCALL OUT_COMMAND ; Call to output command RET OUT_COMMAND: OUT LCD_PORT, R17 ; Output command to the LCD port CBI LCD_PORT, RS ; Clear RS CBI LCD_PORT, RW ; Clear RW SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET // Subroutine to write data to LCD // Input: R17 // Output: None // Description: Writes character to LCD according to ASCII code; for example, R17=0x41 displays the letter A WRITE_DATA: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output first 4 bits RCALL OUT_DATA ; Call to output data POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output last 4 bits RCALL OUT_DATA ; Call to output data RET OUT_DATA: OUT LCD_PORT, R17 ; Output data to the LCD port SBI LCD_PORT, RS ; Set RS for data CBI LCD_PORT, RW ; Clear RW --> Write SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET /* DELAY_US Input: R16 Output: None Description: DELAY R16*100 MICROSEC */ DELAY_US: MOV R15, R16 ; Move R16 to R15 LDI R16, 200 ; Load delay value L1: MOV R14, R16 ; Move delay value to R14 L2: DEC R14 ; Decrement R14 NOP ; No operation BRNE L2 ; Branch if not equal to zero DEC R15 ; Decrement R15 BRNE L1 ; Branch if not equal to zero RET //CONTENT LINE1: .DB "VXL-AVR", $00 ; Define string for line 1 LINE2: .DB "GROUP: 3", $00 ; Define string for line 2

-Switch pressed and count on LCD-

//Declare LCD display .EQU LCD_PORT =PORTA ; Define LCD port .EQU LCD_DDR =DDRA ; Define LCD data direction .EQU RS =0 ; Register Select pin .EQU RW =1 ; Read/Write pin .EQU EN =2 ; Enable pin .DEF ASCII =R19 ; ASCII number to transform (48 is decimal) .DEF UNITS =R20 ; Unit number .DEF TENS =R21 ; Tens number .DEF HUNDS =R22 ; Hundreds number //Declare LED display .EQU LED_PORT =PORTB ; Define LED port .EQU LED_DDR =DDRB ; Define LED data direction //Declare SW .EQU SW_PORT =PORTC ; Define SW port .EQU SW_DDR =DDRC ; Define SW data direction .EQU SW_PIN =PINC ; Define SW pin data .DEF SW_COUNTER =R18 ; Define counter for button //Start at 00 .ORG 00 //Initialize stack pointer LDI R16, LOW(RAMEND) ; Load the low address of RAMEND into R16 OUT SPL, R16 ; Output the low byte to the stack pointer low LDI R16, HIGH(RAMEND) ; Load the high address of RAMEND into R16 OUT SPH, R16 ; Output the high byte to the stack pointer high //Initialize LEDs LDI R16, 0xFF ; Load FF to R16 OUT LED_DDR, R16 ; Load LED_DDR = FF --> OUTPUT LDI R16, 0x00 OUT LED_PORT, R16 ; Turn off LEDS //Initialize Switch CBI SW_DDR, 0 ; Clear DDR bit 0 --> bit 0 is INPUT SBI SW_PORT, 0 ; Set PORT bit 0 --> Pull up register //End initialization //MAIN PROGRAM MAIN: RCALL INIT_LCD // Call the subroutine to initialize the power (POWERUP_LCD_4BIT) for 4-bit LCD mode. Refer to the curriculum for specifics. Do not call this subroutine; otherwise, the LCD will not display anything. RCALL POWERUP_LCD_4BIT ;Use this subroutine to turn on LCD // Call the subroutine to configure the LCD. RCALL INIT_LCD_4BIT ;Configure the LCD // Call the command to set the DDRAM pointer to 0x00. LDI R17, 0x00 ; RCALL WRITE_COMMAND ;Write command to LCD // Below is the code to reference the table and display characters from the first row on the LCD until a 0x00 character is encountered. LDI ZH, HIGH(LINE1 << 1) ; Load high byte of LINE1 address into ZH LDI ZL, LOW(LINE1 << 1) ; Load low byte of LINE1 address into ZL DISPLAY_LINE1: LPM R17, Z+ ; Load character from the program memory into R17 and then count up CPI R17, $00 ; Compare R17 with 0x00 BREQ NEWLINE ; Branch to NEWLINE if equal RCALL WRITE_DATA ; Call to write data to LCD RJMP DISPLAY_LINE1 ; Repeat the process for the next character NEWLINE: // Write command to move to the next line (command to set DDRAM address at 0x40) //LDI R17, 0xC0 //RCALL WRITE_COMMAND LDI ASCII, 48 LDI SW_COUNTER, 0 // Similarly, write code to display on the second row //END LCD_MODULE //LOOP LOOP: //Loop to check the input RCALL CHECK_BUTTON // LDI R17, 0xC0 ;Reset cursor at first slot of 2nd line RCALL WRITE_COMMAND // CPI HUNDS, 0 BRNE H_POS ;Hundreds place CPI TENS, 0 BRNE T_POS ;Tens place CPI UNITS, 0 BRNE U_POS ;Ones place H_POS: MOV R17, HUNDS ADD R17, ASCII CALL WRITE_DATA T_POS: MOV R17, TENS ADD R17, ASCII CALL WRITE_DATA U_POS: MOV R17, UNITS ADD R17, ASCII CALL WRITE_DATA RJMP LOOP //END LOOP ;------------------------------------------------ // Subroutine ;------------------------------------------------ // INIT_LCD // Input: None // Output: None // Description: Preset for LCD before further execution INIT_LCD: LDI R20, $FF ;R20 = 1111 1111 OUT LCD_DDR, R20 ;Load direction = 1 --> DDRA is in output state LDI R20, $00 ;R20 = 0000 0000 OUT LCD_PORT, R20 ;Clear all values of PORTA SBI LCD_PORT, RW ;Set bit to READ CBI LCD_PORT, RS ;Clear bit Register Select CBI LCD_PORT, EN ;Clear bit Enable RET // POWERUP_LCD_4BIT // Input: None // Output: None // Description: Subroutine to power up the LCD POWERUP_LCD_4BIT: //Wait for the LCD to turn on --> need delay LDI R16, 250 ; Load 250 into R16 RCALL DELAY_US ; Call the delay subroutine //Command for 8 bit LDI R17, $28 ; Load command to initialize LCD in 8-bit mode RCALL OUT_COMMAND ; Call to output command //Delay LDI R16, 20 ; Load 50 into R16 RCALL DELAY_US ; Call the delay subroutine // RET // LCD Initialization Subroutine // Input: None // Output: None // Description: Initializes the LCD INIT_LCD_4BIT: LDI R17, $01 ; Command to clear the display RCALL WRITE_COMMAND ; Call to write command LDI R17, $28 ; Command for 4-bit mode, 2 lines, 5x8 dots RCALL WRITE_COMMAND ; Call to write command LDI R17, $0C ; Command to turn on display, cursor off RCALL WRITE_COMMAND ; Call to write command //LDI R17, $06 ; Command to shift cursor right //CALL WRITE_COMMAND ; Call to write command RET // CHECK_BUTTON // Input: None // Output: None // Description: Check the state of button (with debouncing) CHECK_BUTTON: //Check for the press state PRESS_BUTTON: SBIC SW_PIN, 0 ; Skip if button is not pressed RJMP PRESS_BUTTON ; Come back to check button's state LDI R16, 250 ; Load to Delay 10ms RCALL DELAY_US ; Call delay subroutine SBIC SW_PIN, 0 RJMP PRESS_BUTTON //Check for the release state RELEASE_BUTTON: SBIS SW_PIN, 0 ; Skip if button is pressed RJMP RELEASE_BUTTON LDI R16, 250 ; Load to Delay 10ms RCALL DELAY_US ; Call delay subroutine SBIC SW_PIN, 0 RJMP RELEASE_BUTTON // Continue when the button is debounced INC SW_COUNTER ; Press count +1 OUT LED_PORT, SW_COUNTER ; Output to the LED MOV R17, SW_COUNTER RCALL BCD_CONV ; Call BCD converter RET // BCD_CONV // Input: None // Output: None // Description: Convert number to BCD form BCD_CONV: CLR UNITS CLR TENS CLR HUNDS // LDI R16, 10 ; Devide for 10 RCALL DIV10 ; Devide 10 subroutine MOV UNITS, R16 ; Load unit number // LDI R16, 10 ; Devide for 10 RCALL DIV10 ; Devide 10 subroutine MOV TENS, R16 ; Load ten number // MOV HUNDS, R17 // RET // DIV10 // Input: None // Output: None // Description: Divide 10 subroutine DIV10: CLR R15 ; R15: quotient MINUS_ING: SUB R17, R16 ; Subtract R16 from R17 BRCS RESULT ; Branch if carry set (not divisible) INC R15 ; Increment quotient RJMP MINUS_ING ; Continue subtracting RESULT: ADD R17, R16 ; Restore the remainder MOV R16, R17 ; R16: remainder MOV R17, R15 ; R17: quotient RET // Subroutine to write command to LCD // Input: R17 // Output: None // Description: Writes command contained in R17 to the LCD, this goes with OUT_COMMAND WRITE_COMMAND: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output the first 4 bits RCALL OUT_COMMAND ; Call to output command POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output the last 4 bits RCALL OUT_COMMAND ; Call to output command RET OUT_COMMAND: OUT LCD_PORT, R17 ; Output command to the LCD port CBI LCD_PORT, RS ; Clear RS CBI LCD_PORT, RW ; Clear RW SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET // Subroutine to write data to LCD // Input: R17 // Output: None // Description: Writes character to LCD according to ASCII code; for example, R17=0x41 displays the letter A WRITE_DATA: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output first 4 bits RCALL OUT_DATA ; Call to output data POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output last 4 bits RCALL OUT_DATA ; Call to output data RET OUT_DATA: OUT LCD_PORT, R17 ; Output data to the LCD port SBI LCD_PORT, RS ; Set RS for data CBI LCD_PORT, RW ; Clear RW --> Write SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET /* DELAY_US Input: R16 Output: None Description: DELAY R16*100 MICROSEC */ DELAY_US: MOV R15, R16 ; Move R16 to R15 LDI R16, 200 ; Load delay value L1: MOV R14, R16 ; Move delay value to R14 L2: DEC R14 ; Decrement R14 NOP ; No operation BRNE L2 ; Branch if not equal to zero DEC R15 ; Decrement R15 BRNE L1 ; Branch if not equal to zero RET //CONTENT LINE1: .DB "BUTTON COUNTER: ", $00 ; Define string for line 1

-Matrix keypad-

//Declare LCD port .EQU LCD_PORT =PORTA ; Define LCD port .EQU LCD_DDR =DDRA ; Define LCD data direction .EQU RS =0 ; Register Select pin .EQU RW =1 ; Read/Write pin .EQU EN =2 ; Enable pin //Declare LED .EQU LED_PORT =PORTD ; Define LED port = PORTD .EQU LED_DDR =DDRD ; Define LED data direction = DDRD //Declare Matrix keypad .EQU MATRIX_BTN_PORT =PORTB ; Define Matrix keypad port .EQU MATRIX_BTN_DDR =DDRB ; Define matrix keypad data direction .EQU MATRIX_BTN_PIN =PINB ; Define matrix keypad pin data //ELSE .DEF ASCII =R19 ; ASCII number to transform (48 is decimal) .DEF SW_COUNTER =R18 ; Define counter for button //Start .ORG 0 //INITIALIZE LDI R16, LOW(RAMEND) ; Load the low address of RAMEND into R16 OUT SPL, R16 ; Output the low byte to the stack pointer low LDI R16, HIGH(RAMEND) ; Load the high address of RAMEND into R16 OUT SPH, R16 ; Output the high byte to the stack pointer high //End initializing stack pointer //MAIN MAIN: CALL PRESET // Call the subroutine to initialize the power (POWERUP_LCD_4BIT) for 4-bit LCD mode. Refer to the curriculum for specifics. Do not call this subroutine; otherwise, the LCD will not display anything. CALL POWERUP_LCD_4BIT ;Use this subroutine to turn on LCD // Call the subroutine to configure the LCD. CALL INIT_LCD_4BIT ;Configure the LCD // Call the command to set the DDRAM pointer to 0x00. LDI R17, 0x00 ; CALL WRITE_COMMAND ;Write command to LCD // Below is the code to reference the table and display characters from the first row on the LCD until a 0x00 character is encountered. LDI ZH, HIGH(LINE1 << 1) ; Load high byte of LINE1 address into ZH LDI ZL, LOW(LINE1 << 1) ; Load low byte of LINE1 address into ZL DISPLAY_LINE1: LPM R17, Z+ ; Load character from the program memory into R17 and then count up CPI R17, $00 ; Compare R17 with 0x00 BREQ NEWLINE ; Branch to NEWLINE if equal CALL WRITE_DATA ; Call to write data to LCD RJMP DISPLAY_LINE1 ; Repeat the process for the next character NEWLINE: // Write command to move to the next line (command to set DDRAM address at 0x40) LDI SW_COUNTER, 0 LDI ASCII, 48 //END MAIN //LOOP LOOP: LDI R17, 0xC0 CALL WRITE_COMMAND CALL KEY_RD RJMP LOOP //END LOOP // Subroutine ;------------------------------------------------ // PRESET // Input: None // Output: None // Description: Preset for LCD before further execution PRESET: //Setting for LCD LDI R20, $FF ;R20 = 1111 1111 OUT LCD_DDR, R20 ;Load direction = 1 --> DDRA is in output state LDI R20, $00 ;R20 = 0000 0000 OUT LCD_PORT, R20 ;Clear all values of PORTA SBI LCD_PORT, RW ;Set bit to READ CBI LCD_PORT, RS ;Clear bit Register Select CBI LCD_PORT, EN ;Clear bit Enable //Setting for MATRIX KEYPAD LDI R20, 0x0F ;Load R20 = 0000 1111 (Low nibble is for cols, high nibble is for rows) OUT MATRIX_BTN_DDR, R20 SWAP R20 ;Swap R20 --> R20 = 1111 0000 (Set cols to 1 and rows to 0) OUT MATRIX_BTN_PORT, R20 // RET // POWERUP_LCD_4BIT // Input: None // Output: None // Description: Subroutine to power up the LCD POWERUP_LCD_4BIT: //Wait for the LCD to turn on --> need delay LDI R16, 250 ; Load 250 into R16 RCALL DELAY_US ; Call the delay subroutine //Command for 8 bit LDI R17, $28 ; Load command to initialize LCD in 8-bit mode RCALL OUT_COMMAND ; Call to output command //Delay LDI R16, 20 ; Load 50 into R16 RCALL DELAY_US ; Call the delay subroutine // RET // LCD Initialization Subroutine // Input: None // Output: None // Description: Initializes the LCD INIT_LCD_4BIT: LDI R17, $01 ; Command to clear the display CALL WRITE_COMMAND ; Call to write command LDI R17, $28 ; Command for 4-bit mode, 2 lines, 5x8 dots CALL WRITE_COMMAND ; Call to write command LDI R17, $0C ; Command to turn on display, cursor off CALL WRITE_COMMAND ; Call to write command //LDI R17, $06 ; Command to shift cursor right //CALL WRITE_COMMAND ; Call to write command RET // Subroutine to write command to LCD // Input: R17 // Output: None // Description: Writes command contained in R17 to the LCD, this goes with OUT_COMMAND WRITE_COMMAND: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output the first 4 bits RCALL OUT_COMMAND ; Call to output command POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output the last 4 bits RCALL OUT_COMMAND ; Call to output command RET OUT_COMMAND: OUT LCD_PORT, R17 ; Output command to the LCD port CBI LCD_PORT, RS ; Clear RS CBI LCD_PORT, RW ; Clear RW SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET // Subroutine to write data to LCD // Input: R17 // Output: None // Description: Writes character to LCD according to ASCII code; for example, R17=0x41 displays the letter A WRITE_DATA: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output first 4 bits RCALL OUT_DATA ; Call to output data POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output last 4 bits RCALL OUT_DATA ; Call to output data RET OUT_DATA: OUT LCD_PORT, R17 ; Output data to the LCD port SBI LCD_PORT, RS ; Set RS for data CBI LCD_PORT, RW ; Clear RW --> Write SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET /* DELAY_US Input: R16 Output: None Description: DELAY R16*100 MICROSEC */ DELAY_US: MOV R15, R16 ; Move R16 to R15 LDI R16, 200 ; Load delay value L1: MOV R14, R16 ; Move delay value to R14 L2: DEC R14 ; Decrement R14 NOP ; No operation BRNE L2 ; Branch if not equal to zero DEC R15 ; Decrement R15 BRNE L1 ; Branch if not equal to zero RET //CONTENT LINE1: .DB "BUTTON PRESSED: ", $00 ; Define string for line 1 // GET_KEY16: LDI R17, 4 ; R17 = Number of column scans LDI R20, 0xFE ; Start scanning column 0 SCAN_COL: OUT MATRIX_BTN_PORT, R20 IN R19, MATRIX_BTN_PIN ; Read row status IN R19, MATRIX_BTN_PIN ; Read row status again ANDI R19, 0xF0 ; Mask upper 4 bits to get row code CPI R19, 0xF0 ; Check if a key is pressed? BRNE CHK_KEY ; R19 is not F0H, a key is pressed LSL R20 ; Scan next column INC R20 ; Set LSB = 1 DEC R17 ; Decrement the number of column scans BRNE SCAN_COL ; Continue scanning until all columns are checked CLC ; No key pressed, C = 0 CLR R21 ; R21 = 0 NO KEY PRESSED RJMP EXIT ; Exit ;------------------------------------------------- CHK_KEY: SUBI R17, 4 ; Calculate column position NEG R17 ; Offset by 2 to get a positive number SWAP R19 ; Swap to the lower 4 bits of the row code LDI R20, 4 ; R20 holds the scan number for rows SCAN_ROW: ROR R19 ; Rotate right through C to find the 0 bit BRCC SET_FLG ; C=0 indicates row position has a key pressed INC R17 ; If not, increment row position (add 4) INC R17 INC R17 INC R17 DEC R20 BRNE SCAN_ROW ; Continue scanning all 4 rows CLC ; No key pressed, C = 0 RJMP EXIT ; Exit SET_FLG: SEC ; A key is pressed, C = 1 SER R21 ; R21 is not zero, indicating a key is pressed EXIT: RET ;----------------------------------- ;------------------------------------------ ; KEY_RD reads the key status ; Debounce the key press 50 times ; Use GET_KEY16 to identify the pressed key ; Exit when a key is pressed!!! ;------------------------------------------- KEY_RD: LDI R16, 50 ; Number of times to check key press BACK1: RCALL GET_KEY16 ; Call function to detect key press BRCC KEY_RD ; C=0 means no key pressed, loop again DEC R16 ; Decrement the number of key press checks BRNE BACK1 ; Loop until the count reaches zero PUSH R17 LDI R17, $C0 ; Move to row 2, position 1 CALL WRITE_COMMAND POP R17 MOV R24, R17 ; Store the value in R24 OUT LED_PORT, R24 RCALL CHUYEN_ASCII RCALL WRITE_DATA LDI R17, 32 RCALL WRITE_DATA WAIT_1: LDI R16, 50 ; Number of times to check key press BACK2: RCALL GET_KEY16 ; Call function to detect key press BRCS WAIT_1 ; C=1 means a key is still pressed DEC R16 ; Decrement the number of key press checks BRNE BACK2 ; Loop until the count reaches zero ;POP R17 LDI R17, $C0 ; Move to row 2, position 1 CALL WRITE_COMMAND RCALL DISPLAY_FF LDI R24, $FF OUT LED_PORT, R24 RET RET CHUYEN_ASCII: PUSH R18 CPI R17,0x0A BRCS NUM LDI R18, 0x37 RJMP CHAR NUM: LDI R18, 0x30 CHAR: ADD R17, R18 POP R18 RET DISPLAY_FF: PUSH R17 LDI R17, 15 CALL CHUYEN_ASCII RCALL WRITE_DATA LDI R17, 15 CALL CHUYEN_ASCII RCALL WRITE_DATA POP R17 RET

Lab2:

-Delay timer1-

DELAY_1ms: LDI R20, 0x00 STS TCNT1H, R20 STS TCNT1L, R20 LDI R20, LOW(124) ;124 STS OCR1AL, R20 LDI R20, HIGH(124) STS OCR1AH, R20 LDI R20, 0x00 STS TCCR1A, R20 LDI R20, (1 << WGM12) ¦ (1 << CS11) ¦ (1 << CS10);prescale 64 STS TCCR1B, R20 AGAIN: SBIS TIFR1, OCF1A RJMP AGAIN LDI R19, 0x00 STS TCCR1B, R19 STS TCCR1A, R19 LDI R20, 1<<OCF1A OUT TIFR1, R20 RET

-Scan LED-

table_7seg_data: .DB 0XC0, 0XF9,0XA4,0XB0,0X99,0X92,0X82,0XF8,0X80,0X90,0X88,0X8 .DB 0XC6,0XA1,0X86,0X8E ; Lookup table for LED control table_7seg_control: .DB 0b00001110,0b00001101, 0b00001011, 0b00000111 ; J34 connect to PORTD ; nLE0 connect to PB4 ; nLE1 connect to PB5 ; Output: None .equ LED7SEGPORT = PORTD .equ LED7SEGDIR = DDRD .equ LED7SEGLatchPORT = PORTB .equ LED7SEGLatchDIR = DDRB .equ INPUTPORT = PORTA .equ INPUTDDR = DDRA .equ INPUTPIN = PINA .equ nLE0Pin = 4 .equ nLE1Pin = 5 main: RCALL led7seg_portinit LOOP: IN R17, INPUTPIN LDI R18, 1 MUL R17, R18 MOV XL, R0 MOV XH, R1 ;store RCALL NUM_8_BIT_TO_BCD RCALL SCAN_4LA RJMP LOOP led7seg_portinit: push r20 ldi r20, 0b11111111 ; SET led7seg PORT as output out LED7SEGDIR, r20 in r20, LED7SEGLatchDIR ; read the Latch Port direction register ori r20, (1<<nLE0Pin) ¦ (1 << nLE1Pin) out LED7SEGLatchDIR,r20 LDI R20, 0x00 OUT INPUTDDR, R20 LDI R20, 0xFF OUT INPUTPORT, R20 pop r20 ret ; Display a value on a 7-segment LED using a lookup table ; Input: R27 contains the value to display ; R26 contain the LED index (3..0) ; J34 connect to PORTD ; nLE0 connect to PB4 ; nLE1 connect to PB5 ; Output: None display_7seg: push r16 ; Save the temporary register ; Look up the 7-segment code for the value in R18 ; Note that this assumes a common anode display, where a HIGH output turns OFF the segment ; If using a common cathode display, invert the values in the table above ldi zh,high(table_7seg_data<<1) ; ldi zl,low(table_7seg_data<<1) ; clr r16 add r30, r27 adc r31,r16 lpm r16, z out LED7SEGPORT,r16 sbi LED7SEGLatchPORT,nLE0Pin nop cbi LED7SEGLatchPORT,nLE0Pin ldi zh,high(table_7seg_control<<1) ; ldi zl,low(table_7seg_control<<1) ; clr r16 add r30, r26 adc r31,r16 lpm r16, z out LED7SEGPORT,r16 sbi LED7SEGLatchPORT,nLE1Pin nop cbi LED7SEGLatchPORT,nLE1Pin pop r16 ; Restore the temporary register ret ; Return from the function NUM_8_BIT_TO_BCD: CLR R21 ; Clear units place CLR R22 ; Clear tens place CLR R23 ; Clear hundreds place CLR R24 LDI R16, 10 ; Load divisor (10) RCALL DIV10 ; Call division subroutine MOV R21, R16 ; R20: units place LDI R16, 10 ; Load divisor (10) RCALL DIV10 ; Call division subroutine MOV R22, R16 LDI R16, 10 ; Load divisor (10) RCALL DIV10 ; Call division subroutine MOV R23, R16 ; R21: tens place MOV R24, XL ; R22: hundreds place RET ;------------------------------------------------------------ ; INPUT: R17, R16 = 10 ; OUTPUT: R16: remainder, R17: quotient ; DESCRIPTION: Divide R17 by R16 ;------------------------------------------------------------ DIV10: CLR R15 ; R15: quotient TIEP_TUC_TRU: ;SUB R17, R16 ; Subtract R16 from R17 SBIW X, 10 BRCS DUARA_KETQUA ; Branch if carry set (not divisible) INC R15 ; Increment quotient RJMP TIEP_TUC_TRU ; Continue subtracting DUARA_KETQUA: ;ADD R17, R16 ; Restore the remainder ADIW X, 10 MOV R16, XL ; R16: remainder MOV XL, R15 ; R17: quotient RET SCAN_4LA: PUSH R17 ; RCALL BCD_UNP LDI R25, 4; the value of segment Red using LP: LDI R26, 0xFF; turn of all led RCALL display_7seg CPI R25, 4 BREQ DISPLAY_units CPI R25, 3 BREQ DISPLAY_ten_units CPI R25, 2 BREQ DISPLAY_hundred_units CPI R25, 1 BREQ DISPLAY_thousand_units DISPLAY_units: MOV R27, R21 LDI R26, 0 RCALL display_7seg RJMP Dlscan DISPLAY_ten_units: MOV R27, R22 LDI R26, 1 RCALL display_7seg RJMP Dlscan DISPLAY_hundred_units: MOV R27, R23 LDI R26, 2 RCALL display_7seg RJMP Dlscan DISPLAY_thousand_units: MOV R27, R24 LDI R26, 3 RCALL display_7seg RJMP Dlscan Dlscan: RCALL DELAY_1ms SEC DEC R25 BRNE LP POP R17 RET DELAY_1ms: PUSH R16 PUSH R17 LOOP_AGAIN: LDI R17, 250 LOOP_2: LDI R16, 8 LOOP_1: NOP DEC R16 BRNE LOOP_1 DEC R17 BRNE LOOP_2 POP R17 POP R16 RET

-Matrix LED display-

.include "m324padef.inc" ; Include Atmega324pa definitions .org 0x0000 ; interrupt vector table rjmp reset_handler ; reset .org 0x001A rjmp timer1_COMP_ISR .equ clearSignalPort = PORTB ; Set clear signal port to PORTB .equ clearSignalPin = 3 ; Set clear signal pin to pin 3 of PORTB .equ shiftClockPort = PORTB ; Set shift clock port to PORTB .equ shiftClockPin = 2 ; Set shift clock pin to pin 2 of PORTB .equ latchPort = PORTB ; Set latch port to PORTB .equ latchPin = 1 ; Set latch pin to pin 1 of PORTB .equ shiftDataPort = PORTB ; Set shift data port to PORTB .equ shiftDataPin = 0 ; Set shift data pin to pin 0 of PORTB // ;Lookup table for collumn control ledmatrix_col_control: .DB 0x80, 0x40, 0x20, 0x10, 0x08, 0x04, 0x02, 0x01 ; Lookup table for font ledmatrix_Font_A: .DB 0b11111100, 0b00010010, 0b00010001, 0b00010001, 0b00010010, 0b11111100, 0b00000000, 0b00000000 .equ LEDMATRIXPORT = PORTD .equ LEDMATRIXDIR = DDRD .dseg .org SRAM_START ;starting address is 0x100 LedMatrixBuffer : .byte 8 LedMatrixColIndex : .byte 1 .cseg .align 2 reset_handler: ; initialize stack pointer ldi r16, high(RAMEND) out SPH, r16 ldi r16, low(RAMEND) out SPL, r16 call shiftregister_initport call shiftregister_cleardata call initTimer1CTC ; enable global interrupts sei call ledmatrix_portinit main: jmp main ; Initialize ports as outputs shiftregister_initport: push r24 ldi r24, (1<<clearSignalPin)¦(1<<shiftClockPin)¦(1<<latchPin)¦(1<<shiftDataPin); out DDRB, r24 ; Set DDRB to output pop r24 ret shiftregister_cleardata: cbi clearSignalPort, clearSignalPin ; Set clear signal pin to low nop ; Wait for a short time sbi clearSignalPort, clearSignalPin ; Set clear signal pin to high ret ; Shift out data ;shift out R27 to bar led shiftregister_shiftoutdata: push r18 cbi shiftClockPort, shiftClockPin ; ldi r18, 8 ; Shift 8 bits shiftloop: sbrc r27, 7 ; Check if the MSB of shiftData is 1 sbi shiftDataPort, shiftDataPin ; Set shift data pin to high sbi shiftClockPort, shiftClockPin ; Set shift clock pin to high lsl r27 ; Shift left cbi shiftClockPort, shiftClockPin ; Set shift clock pin to low cbi shiftDataPort, shiftDataPin ; Set shift data pin to low dec r18 brne shiftloop ; Latch data sbi latchPort, latchPin ; Set latch pin to high cbi latchPort, latchPin ; Set latch pin to low pop r18 ret ; J38 connect to PORTD ; clear signal pin to pin 0 of PORTB ; shift clock pin to pin 1 of PORTB ; latch pin to pin 0 of PORTB ; shift data pin to pin 3 of PORTB ; Output: None ledmatrix_portinit: push r20 push r21 ldi r20, 0b11111111 ; SET port as output out LEDMATRIXDIR, r20 ldi r20,0 ;col index start at 0 ldi r31,high(LedMatrixColIndex) ldi r30,low(LedMatrixColIndex) st z,r20 ldi r20,0 ldi r31,high(ledmatrix_Font_A << 1) ;Z register point to fontA value ldi r30,low(ledmatrix_Font_A << 1) ldi r29,high(LedMatrixBuffer) ; Y register point to fontA value ldi r28,low(LedMatrixBuffer) ldi r20,8 ledmatrix_portinit_loop: ;copy font to display buffer lpm r21,z+ st y+,r21 dec r20 cpi r20,0 brne ledmatrix_portinit_loop pop r21 pop r20 ret ; Display a Collumn of Led Matrix ; Input: R27 contains the value to display ; R26 contain the Col index (3..0) ; Output: None ledmatrix_display_col: push r16 ; Save the temporary register push r27 clr r16 out LEDMATRIXPORT,r16 call shiftregister_shiftoutdata ldi r31,high(ledmatrix_col_control << 1) ldi r30,low(ledmatrix_col_control << 1) clr r16 add r30,r26 adc r31,r16 lpm r27,z out LEDMATRIXPORT,r27 pop r27 pop r16 ; Restore the temporary register ret ; Return from the function initTimer1CTC: push r16 ldi r16, high(2500) ; Load the high byte into the temporary register sts OCR1AH, r16 ; Set the high byte of the timer 1 compare value ldi r16, low(2500) ; Load the low byte into the temporary register sts OCR1AL, r16 ; Set the low byte of the timer 1 compare value ldi r16, (1 << CS10)¦ (1<< WGM12) ; Load the value 0b00000101 into the temporary register sts TCCR1B, r16 ; ldi r16, (1 << OCIE1A); Load the value 0b00000010 into the temporary register sts TIMSK1, r16 ; Enable the timer 1 compare A interrupt pop r16 ret timer1_COMP_ISR: push r16 push r26 push r27 ldi r31,high(LedMatrixColIndex) ldi r30,low(LedMatrixColIndex) ld r16,z mov r26,r16 ldi r31,high(LedMatrixBuffer) ldi r30,low(LedMatrixBuffer) add r30,r16 clr r16 adc r31,r16 ld r27,z call ledmatrix_display_col inc r26 cpi r26,8 brne timer1_COMP_ISR_CONT ldi r26,0 ;if r26 = 8, reset to 0 timer1_COMP_ISR_CONT: ldi r31,high(LedMatrixColIndex) ldi r30,low(LedMatrixColIndex) st z,r26 pop r27 pop r26 pop r16 reti

Lab3:

-Timer 1 overflow interrupt-

//Delay 0.5ms .ORG 0 RJMP RESET .ORG 0X001E // TIMER1_OVF RJMP TIMER1_ISR .ORG 0X0100 RESET: .EQU DELAYTIME = -500 // LDI R16,HIGH(RAMEND) OUT SPH,R16 LDI R16,LOW(RAMEND) OUT SPL,R16 // LDI R17,1 // SBI DDRC,0 ; PC0 OUTPUT LDI R16,HIGH(DELAYTIME) STS TCNT1H,R16 LDI R16,LOW(DELAYTIME) STS TCNT1L,R16 // LDI R16,0X00 STS TCCR1A,R16 ;TIMER1 MODE NOR LDI R16,(1<<CS11) ; TIMER1 CHẠY, HỆ SỐ CHIA N=8 STS TCCR1B,R16 SEI ; CHO PHÉP NGẮT TOÀN CỤC LDI R16, (1<<TOIE1) ; CHO PHÉP NGẮT TIMER1 TRÀN STS TIMSK1,R16 MAIN: RJMP MAIN TIMER1_ISR: IN R16,PORTC EOR R16,R17 OUT PORTC,R16 // LDI R16,HIGH(DELAYTIME) STS TCNT1H,R16 LDI R16,LOW(DELAYTIME) STS TCNT1L,R16 LDI R16,(1<<CS11) STS TCCR1B,R16 RETI

-Timer1 CTC interrupt-

//Delay 5ms .ORG 0 RJMP RESET .ORG 0X001A // TIMER1_COMPA RJMP TIMER1_ISR .ORG 0X40 RESET: .DEF COUNTER = R20 .EQU DELAYTIME = 999 // LDI R16,HIGH(RAMEND) OUT SPH,R16 LDI R16,LOW(RAMEND) OUT SPL,R16 // LDI R17,1 LDI COUNTER,5 SBI DDRC,0 // LDI R16,HIGH(DELAYTIME) STS OCR1AH,R16 LDI R16,LOW(DELAYTIME) STS OCR1AL,R16 LDI R16, 0X00 ; TIMER1 MODE CTC4 STS TCCR1A,R16 LDI R16,(1<<CS11)¦(1<<WGM12) STS TCCR1B,R16 SEI LDI R16, (1<<OCF1A) STS TIMSK1,R16 MAIN: RJMP MAIN TIMER1_ISR: DEC COUNTER BRNE OUT_ISR IN R16,PORTC EOR R16,R17 OUT PORTC,R16 LDI COUNTER,5 OUT_ISR: RETI

-Scan LEDs with interrupt-

.EQU OUTPORT = PORTD .EQU OUTDDR = DDRD .EQU INPIN = PIND .EQU OUTPORT_ENABLE = PORTB .EQU DDR_ENABLE = DDRB .equ LED7SEGLatchPORT = PORTB .equ LED7SEGLatchDIR = DDRB .equ nLE0Pin = 4 .equ nLE1Pin = 5 .ORG 0 RJMP MAIN .ORG 0x001A RJMP TIMER_1_OCPA_ISR .ORG 0x0040 MAIN: LDI R16, HIGH(RAMEND) OUT SPH, R16 LDI R16, LOW(RAMEND) OUT SPL, R16 LDI R16, 0x0F RCALL led7seg_portinit LDI R16, 0xFF; OUT OUTDDR, R16; LDI R17, 0x00 STS OCR1AH, R17; LDI R17, 0xFF STS OCR1AL, R17; LDI R17, 0x02 STS TCCR1A, R17 LDI R17, 0x05 STS TCCR1B, R17 SEI LDI R17, (1<<OCIE1A) STS TIMSK1, R17 LDI R25, 0x00 LDI R18, 4; the value of segment Red using START: mov R17, R25 RCALL SCAN_4LA RJMP START TIMER_1_OCPA_ISR: PUSH R17 CPI R18, 0 BRNE NEXT LDI R18, 4 NEXT: LDI R17, 0x00 STS TCCR1B, R17 LDI R17, 0x00 STS OCR1AH, R17; LDI R17, 0xFF STS OCR1AL, R17; LDI R17, 0x05 STS TCCR1B, R17 POP R17 DEC R18 ; code th m PC0 ENDING: RETI SCAN_4LA: PUSH R17 ; RCALL BCD_UNP LOOP: LDI R17, 0xFF; turn of all led OUT OUTPORT, R17 CPI R18, 3 BRNE DISPLAY_ten_units LDI R26, 0 LDI R27, 4 RCALL display_7seg OUT OUTPORT, R17 RJMP Dlscan DISPLAY_ten_units: CPI R18, 2 BRNE DISPLAY_hundred_units LDI R26, 1 LDI R27, 3 RCALL display_7seg OUT OUTPORT, R17 RJMP Dlscan DISPLAY_hundred_units: CPI R18, 1 BRNE DISPLAY_thousand_units LDI R26, 2 LDI R27, 2 RCALL display_7seg OUT OUTPORT, R17 RJMP Dlscan DISPLAY_thousand_units: CPI R18, 0 BRNE Dlscan LDI R26, 3 LDI R27, 1 RCALL display_7seg OUT OUTPORT, R17 Dlscan: POP R17 RET led7seg_portinit: push r20 ldi r20, 0b11111111 ; SET led7seg PORT as output out OUTDDR, r20 in r20, LED7SEGLatchDIR ; read the Latch Port direction register ori r20, (1<<nLE0Pin) ¦ (1 << nLE1Pin) out LED7SEGLatchDIR,r20 pop r20 ret display_7seg: push r16 ; Save the temporary register ; Look up the 7-segment code for the value in R18 ; Note that this assumes a common anode display, where a HIGH output turns OFF the segment ; If using a common cathode display, invert the values in the table above ldi zh,high(TAB_7SA<<1) ; ldi zl,low(TAB_7SA<<1) ; clr r16 add r30, r27 adc r31,r16 lpm r16, z out OUTPORT,r16 sbi LED7SEGLatchPORT,nLE0Pin nop cbi LED7SEGLatchPORT,nLE0Pin ldi zh,high(table_7seg_control<<1) ; ldi zl,low(table_7seg_control<<1) ; clr r16 add r30, r26 adc r31,r16 lpm r16, z out OUTPORT,r16 sbi LED7SEGLatchPORT,nLE1Pin nop cbi LED7SEGLatchPORT,nLE1Pin pop r16 ; Restore the temporary register ret ; Return from the function TAB_7SA: .DB 0xC0, 0xF9, 0xA4, 0xB0, 0x99, 0x92 .DB 0x82, 0xF8, 0x80, 0x90, 0x88, 0x83 .DB 0xC6, 0xA1, 0x86, 0x8E table_7seg_control: .DB 0b00001110,0b00001101, 0b00001011, 0b00000111

Lab4:

-UART get and print out-

.ORG 0x00 RJMP MAIN .ORG 0x40 MAIN: RCALL USART_INIT START: RCALL USART_TRANS RCALL USART_REC RJMP START USART_INIT: LDI R16, (1<<TXEN0)¦(1<<RXEN0) STS UCSR0B, R16 LDI R16, (1<<UCSZ01)¦(1<<UCSZ00) STS UCSR0C, R16 LDI R16, 0x00 STS UBRR0H, R16 LDI R16, 51 STS UBRR0L, R16 RET USART_TRANS: LDS R17, UCSR0A SBRS R17, UDRE0 RJMP USART_TRANS STS UDR0, R16 ; If UDR0 is empty it wil transmit the data RET USART_TRANS_9BIT: LDS R18, UCSR0A SBRS R18, UDRE0 RJMP USART_TRANS_9BIT LDS R18, UCSR0B ANDI R18, 0xFE SBRC R17, 0; Bit 9 store in R17 ORI R18, 1 STS UCSR0B, R18 STS UDR0, R16 RET USART_REC: LDS R17, UCSR0A SBRS R17, RXC0 ; check having receive data RJMP USART_REC LDS R16, UDR0 RET USART_REC_9BIT: LDS R17, UCSR0A SBRS R17, RXC0 RJMP USART_REC_9BIT LDS R18, UCSR0A ; read the bit status value LDS R17, UCSR0B ;read the bit 9 LDS R16, UDR0 ANDI R18, (1<<FE0)¦(1<<DOR0)¦(1<<UPE0) BREQ USART_REC_NO_ERROR LDI R17, HIGH(-1) LDI R16, LOW(-1) RJMP EXIT USART_REC_NO_ERROR: LSR R17 ANDI R17, 0x01 EXIT: RET

-Input character (UART) and print out on LCD-(UART interrupt) (4-2/ex1)

; Replace with your application code ; Replace with your application code .EQU LCD_PORT=PORTA ; Define LCD port .EQU LCD_DDR=DDRA ; Define LCD data direction .EQU RS=0 ; Register Select pin .EQU RW=1 ; Read/Write pin .EQU EN=2 ; Enable pin ; J34 connect to PORTC ; nLE0 connect to PB4 ; nLE1 connect to PB5 ; Output: None .equ LED7SEGPORT = PORTC .equ LED7SEGDIR = DDRC .equ LED7SEGLatchPORT = PORTB .equ LED7SEGLatchDIR = DDRB .equ nLE0Pin = 4 .equ nLE1Pin = 5 .ORG 0x00 RJMP MAIN .ORG 0x0028 RJMP USART0_RX_ISR .ORG 0X40 MAIN: LDI R16, LOW(RAMEND) ; Load the low address of RAMEND into R16 OUT SPL, R16 ; Output the low byte to the stack pointer low LDI R16, HIGH(RAMEND) ; Load the high address of RAMEND into R16 OUT SPH, R16 ; Output the high byte to the stack pointer high /* Connect port A to the LCD: RS=PA0, RW=PA1, EN=PA2, Data[4:7] = PA4:PA7 => Declare PA0, PA1, PA2, PA4, PA5, PA6, PA7 as output, and initialize values as follows: Data = 0, RS=0, RW=1, EN=0. */ LDI R16, 0xFF OUT LCD_DDR, R16 LDI R16, 0x00 OUT LCD_PORT, R16 CBI LCD_PORT, RS SBI LCD_PORT, RW CBI LCD_PORT, EN // Call the subroutine to initialize the power (POWERUP_LCD_4BIT) for 4-bit LCD mode. Refer to the curriculum for specifics. Do not call this subroutine; otherwise, the LCD will not display anything. call POWERUP_LCD_4BIT // Call the subroutine to configure the LCD. call INIT_LCD_4BIT CALL led7seg_portinit // Call the command to set the DDRAM pointer to 0x00. LDI R17, 0x00 call WRITE_COMMAND // Call the command to set the DDRAM pointer to 0x00. LDI R17, 0x00 call WRITE_COMMAND // Below is the code to reference the table and display characters from the first row on the LCD until a 0x00 character is encountered. LDI ZH, HIGH(LINE1 << 1) ; Load high byte of LINE1 address into ZH LDI ZL, LOW(LINE1 << 1) ; Load low byte of LINE1 address into ZL DISPLAY_LINE1: LPM R17, Z+ ; Load character from the program memory into R17 CPI R17, $00 ; Compare R17 with 0x00 BREQ NEWLINE ; Branch to NEWLINE if equal CALL WRITE_DATA ; Call to write data to LCD RJMP DISPLAY_LINE1 ; Repeat the process for the next character NEWLINE: // Similarly, write code to display on the second row ; R18 stores the number of times the button is pressed LDI R18, 0 ; R19 is a constant for converting to ASCII; note that the count from 0, 1, ... displayed on the LCD needs to be incremented by 48 (decimal). LDI R19, 48 CALL USART_INIT // Similarly, write code to display on the second row LOOP: // This part of the code loops infinitely without any actions, as everything only needs to run once. LDI R17, 0xC0 call WRITE_COMMAND SBRC R6, 0 CALL WRITE_LCD MOV R4, XL MOV R5, XH RCALL NUM_8_BIT_TO_BCD RCALL SCAN_4LA RJMP LOOP // Subroutine WRITE_LCD: MOV R17, R2 CALL WRITE_DATA ADIW X, 10 LDI R30, 0x00 MOV R6, R30 RET ;------------------------------------------------ // POWERUP_LCD_4BIT // Input: None // Output: None // Description: Subroutine to power up the LCD POWERUP_LCD_4BIT: LDI R16, 250 ; Load 250 into R16 RCALL DELAY_US ; Call the delay subroutine LDI R16, 250 ; Load 250 into R16 RCALL DELAY_US ; Call the delay subroutine LDI R17, $30 ; Load command to initialize LCD in 8-bit mode RCALL OUT_COMMAND ; Call to output command LDI R16, 50 ; Load 50 into R16 RCALL DELAY_US ; Call the delay subroutine LDI R17, $30 ; Load command to initialize LCD in 8-bit mode again RCALL OUT_COMMAND ; Call to output command LDI R16, 2 ; Load delay RCALL DELAY_US ; Call the delay subroutine LDI R17, $20 ; Load command for 4-bit mode RCALL OUT_COMMAND ; Call to output command RET // LCD Initialization Subroutine // Input: None // Output: None // Description: Initializes the LCD INIT_LCD_4BIT: LDI R17, $28 ; Command for 4-bit mode, 2 lines, 5x8 dots CALL WRITE_COMMAND ; Call to write command LDI R17, $01 ; Command to clear the display CALL WRITE_COMMAND ; Call to write command LDI R17, $0C ; Command to turn on display, cursor off CALL WRITE_COMMAND ; Call to write command LDI R17, $06 ; Command to shift cursor right CALL WRITE_COMMAND ; Call to write command RET // Subroutine to write command to LCD // Input: R17 // Output: None // Description: Writes command contained in R17 to the LCD. WRITE_COMMAND: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output the first 4 bits RCALL OUT_COMMAND ; Call to output command POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output the last 4 bits RCALL OUT_COMMAND ; Call to output command RET // Subroutine to write data to LCD // Input: R17 // Output: None // Description: Writes character to LCD according to ASCII code; for example, R17=0x41 displays the letter A WRITE_DATA: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output first 4 bits RCALL OUT_DATA ; Call to output data POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output last 4 bits RCALL OUT_DATA ; Call to output data RET OUT_COMMAND: OUT LCD_PORT, R17 ; Output command to the LCD port CBI LCD_PORT, RS ; Clear RS CBI LCD_PORT, RW ; Clear RW SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET OUT_DATA: OUT LCD_PORT, R17 ; Output data to the LCD port SBI LCD_PORT, RS ; Set RS for data CBI LCD_PORT, RW ; Clear RW SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET /* DELAY_US Input: R16 Output: None Description: DELAY R16*100 MICROSEC */ DELAY_US: MOV R15, R16 ; Move R16 to R15 LDI R16, 100 ; Load delay value L1: MOV R14, R16 ; Move delay value to R14 L2: DEC R14 ; Decrement R14 NOP ; No operation BRNE L2 ; Branch if not equal to zero DEC R15 ; Decrement R15 BRNE L1 ; Branch if not equal to zero RET LINE1: .DB "The count is ",$00 USART_INIT: LDI R16, 0x00 STS UBRR0H, R16 LDI R16, 51 STS UBRR0L, R16 LDI R16, (1<<UCSZ00)¦(1<<UCSZ01) STS UCSR0C, R16 LDI R16, (1<<RXEN0)¦(1<<RXCIE0)¦(1<<TXEN0) STS UCSR0B, R16 SEI RET USART0_RX_ISR: IN R17, SREG; store SREG PUSH R17 USART_REC: LDS R17, UCSR0A SBRS R17, RXC0 ; kiee tra da nhan chua RJMP USART_REC LDS R16, UDR0 CALL USART_TRANS EXIT_RX: POP R17 OUT SREG, R17 MOV R2, R16 LDI R30, 0x01 MOV R6, R30 RETI USART_TRANS: PUSH R17 LDS R17, UCSR0A SBRS R17, UDRE0 RJMP USART_TRANS STS UDR0, R16 ; Khi UDR0 trong thi chep du lieu can phat vao UDR0 POP R17 RET ;----------------------LED Segment------------------------------------- led7seg_portinit: push r20 ldi r20, 0b11111111 ; SET led7seg PORT as output out LED7SEGDIR, r20 in r20, LED7SEGLatchDIR ; read the Latch Port direction register ori r20, (1<<nLE0Pin) ¦ (1 << nLE1Pin) out LED7SEGLatchDIR,r20 pop r20 ret ; Display a value on a 7-segment LED using a lookup table ; Input: R27 contains the value to display ; R26 contain the LED index (3..0) ; J34 connect to PORTD ; nLE0 connect to PB4 ; nLE1 connect to PB5 ; Output: None display_7seg: push r16 ; Save the temporary register ; Look up the 7-segment code for the value in R18 ; Note that this assumes a common anode display, where a HIGH output turns OFF the segment ; If using a common cathode display, invert the values in the table above ldi zh,high(table_7seg_data<<1) ; ldi zl,low(table_7seg_data<<1) ; clr r16 add r30, r27 adc r31,r16 lpm r16, z out LED7SEGPORT,r16 sbi LED7SEGLatchPORT,nLE0Pin nop cbi LED7SEGLatchPORT,nLE0Pin ldi zh,high(table_7seg_control<<1) ; ldi zl,low(table_7seg_control<<1) ; clr r16 add r30, r26 adc r31,r16 lpm r16, z out LED7SEGPORT,r16 sbi LED7SEGLatchPORT,nLE1Pin nop cbi LED7SEGLatchPORT,nLE1Pin pop r16 ; Restore the temporary register ret ; Return from the function NUM_8_BIT_TO_BCD: CLR R21 ; Clear units place CLR R22 ; Clear tens place CLR R23 ; Clear hundreds place CLR R24 LDI R16, 10 ; Load divisor (10) RCALL DIV10 ; Call division subroutine MOV R21, R16 ; R20: units place LDI R16, 10 ; Load divisor (10) RCALL DIV10 ; Call division subroutine MOV R22, R16 ; R22: hundred places LDI R16, 10 ; Load divisor (10) RCALL DIV10 ; Call division subroutine MOV R23, R16 ; R23: hundred place MOV R24, XL ; R24: Thousand place RET ;------------------------------------------------------------ ; INPUT: R17, R16 = 10 ; OUTPUT: R16: remainder, R17: quotient ; DESCRIPTION: Divide R17 by R16 ;------------------------------------------------------------ DIV10: CLR R15 ; R15: quotient TIEP_TUC_TRU: ;SUB R17, R16 ; Subtract R16 from R17 SBIW X, 10 BRCS DUARA_KETQUA ; Branch if carry set (not divisible) INC R15 ; Increment quotient RJMP TIEP_TUC_TRU ; Continue subtracting DUARA_KETQUA: ;ADD R17, R16 ; Restore the remainder ADIW X, 10 MOV R16, XL ; R16: remainder MOV XL, R15 ; R17: quotient RET SCAN_4LA: PUSH R17 ; RCALL BCD_UNP LDI R25, 4; the value of segment Red using LP: LDI R26, 0xFF; turn of all led RCALL display_7seg CPI R25, 4 BREQ DISPLAY_units CPI R25, 3 BREQ DISPLAY_ten_units CPI R25, 2 BREQ DISPLAY_hundred_units CPI R25, 1 BREQ DISPLAY_thousand_units DISPLAY_units: MOV R27, R21 LDI R26, 0 RCALL display_7seg RJMP Dlscan DISPLAY_ten_units: MOV R27, R22 LDI R26, 1 RCALL display_7seg RJMP Dlscan DISPLAY_hundred_units: MOV R27, R23 LDI R26, 2 RCALL display_7seg RJMP Dlscan DISPLAY_thousand_units: MOV R27, R24 LDI R26, 3 RCALL display_7seg RJMP Dlscan Dlscan: RCALL DELAY_US SEC DEC R25 BRNE LP MOV XL, R4 MOV XH, R5 POP R17 RET table_7seg_data: .DB 0XC0, 0XF9,0XA4,0XB0,0X99,0X92,0X82,0XF8,0X80,0X90,0X88,0X8 .DB 0XC6,0XA1,0X86,0X8E ; Lookup table for LED control table_7seg_control: .DB 0b00001110,0b00001101, 0b00001011, 0b00000111

-SPI with shift register & BARLED-

.EQU OUTPORT = PORTB .EQU OUTPORT_DDR = DDRB .EQU MOSI = 5 .EQU MISO = 6 .EQU SCK = 7 .EQU SS = 4 .EQU ClearSignalPin = 1 .EQU LatchSignalPin = 3 .ORG 0x00 RJMP MAIN .ORG 0x40 MAIN: LDI R16, HIGH(RAMEND) OUT SPH, R16 LDI R16, LOW(RAMEND) OUT SPL, R16 SBI OUTPORT_DDR, ClearSignalPin SBI OUTPORT_DDR, LatchSignalPin LDI R16, 0x00 OUT OUTPORT, R16 LDI R16, (1<<SCK)¦(1<<MOSI)¦(1<<SS) OUT DDRB, R16 // LDI R16, (1<<SPE0)¦(1<<MSTR0)¦(1<<SPR00) OUT SPCR0, R16 CBI PORTB, SS CALL USART_INIT START: RCALL USART_REC MOV R17, R16 RCALL USART_TRANS SBI PORTB, SS RCALL SPI_TRANS CBI PORTB, SS RJMP START SPI_TRANS: ;LDI R17, 0x05 OUT SPDR0, R17; ghi data ra SPI WAIT_SPI: IN R16, SPSR0; doc co SPIF0 SBRS R16, SPIF0;co SPIF0=1 truyen SPI xong RJMP WAIT_SPI; cho co SPIF0 = 1 IN R18, SPDR0 RET USART_INIT: LDI R16, (1<<TXEN0)¦(1<<RXEN0) STS UCSR0B, R16 LDI R16, (1<<UCSZ01)¦(1<<UCSZ00) STS UCSR0C, R16 LDI R16, 0x00 STS UBRR0H, R16 LDI R16, 51 STS UBRR0L, R16 RET USART_TRANS: PUSH R17 LDS R17, UCSR0A SBRS R17, UDRE0 RJMP USART_TRANS STS UDR0, R16 ; Khi UDR0 trong thi chep du lieu can phat vao UDR0 POP R17 RET USART_TRANS_9BIT: LDS R18, UCSR0A SBRS R18, UDRE0 RJMP USART_TRANS_9BIT LDS R18, UCSR0B ANDI R18, 0xFE SBRC R17, 0; Bit 9 store in R17 ORI R18, 1 STS UCSR0B, R18 STS UDR0, R16 RET USART_REC: LDS R17, UCSR0A SBRS R17, RXC0 ; kiee tra da nhan chua RJMP USART_REC LDS R16, UDR0 RET USART_REC_9BIT: LDS R17, UCSR0A SBRS R17, RXC0 RJMP USART_REC_9BIT LDS R18, UCSR0A ; Doc bit trang thai LDS R17, UCSR0B ; Doc bit data thu 9 LDS R16, UDR0 ANDI R18, (1<<FE0)¦(1<<DOR0)¦(1<<UPE0) BREQ USART_REC_NO_ERROR LDI R17, HIGH(-1) LDI R16, LOW(-1) RJMP EXIT USART_REC_NO_ERROR: LSR R17 ANDI R17, 0x01 EXIT: RET DELAY_US: LDI R28, 200 ; Load delay value MOV R15, R28 ; Move R16 to R15 LDI R28, 200 ; Load delay value L1: MOV R14, R28 ; Move delay value to R14 L2: DEC R14 ; Decrement R14 NOP ; No operation BRNE L2 ; Branch if not equal to zero DEC R15 ; Decrement R15 BRNE L1 ; Branch if not equal to zero RET

Lab5:

-ADC (connect ADC1 and display the value out the LCD)-(Lab5/ex1)-

.EQU ADC_PORT=PORTA .EQU ADC_DR=DDRA .EQU ADC_IN=PINA .EQU TOP_VALUE = 31249 ; Top value for 1-second delay (OCR1A) .EQU COMPB_VALUE = 31249 ; Compare Match B value (OCR1B) .EQU LCD_PORT=PORTB ; Define LCD port .EQU LCD_DDR=DDRB ; Define LCD data direction .EQU RS=0 ; Register Select pin .EQU RW=1 ; Read/Write pin .EQU EN=2 ; Enable pin .ORG 0 RJMP MAIN .ORG 0x001C RJMP TIMER1_COMPB_ISR .ORG 0X40 MAIN: ;LDI R16,0xFF ;OUT ADC_PORT,R16 ;ENABLE PULL UP RES LDI R16,HIGH(RAMEND);??a stack lên vùng ?/c cao OUT SPH,R16 LDI R16,LOW(RAMEND) OUT SPL,R16 ldi r16,0xFF out LCD_DDR,r16 ldi r16,0x02 out LCD_PORT,r16 // Call the subroutine to initialize the power (POWERUP_LCD_4BIT) for 4-bit LCD mode. Refer to the curriculum for specifics. Do not call this subroutine; otherwise, the LCD will not display anything. rcall POWERUP_LCD_4BIT // Call the subroutine to configure the LCD. rcall INIT_LCD_4BIT // Call the command to set the DDRAM pointer to 0x00. ldi r17,0x80 call WRITE_COMMAND // Below is the code to reference the table and display characters from the first row on the LCD until a 0x00 character is encountered. LDI ZH, HIGH(LINE1 << 1) ; Load high byte of LINE1 address into ZH LDI ZL, LOW(LINE1 << 1) ; Load low byte of LINE1 address into ZL DISPLAY_LINE1: LPM R17, Z+ ; Load character from the program memory into R17 CPI R17, $00 ; Compare R17 with 0x00 BREQ NEWLINE ; Branch to NEWLINE if equal CALL WRITE_DATA ; Call to write data to LCD RJMP DISPLAY_LINE1 ; Repeat the process for the next character NEWLINE: // Configure UART, configure the ADC pins as input, configure other in/out pins as per the requirements. //(Todo: Write UART configuration code) RCALL USART_INIT // Configure the timer, select the appropriate timer and mode, and configure the auto trigger settings. //(Todo: Write Timer configuration code) RCALL TIMER1_INIT ; Configure ADC LDI R16, (1<<REFS0) ; AVcc with external capacitor at AREF STS ADMUX, R16 LDI R16, (1<<ADEN) ¦ (1<<ADSC)¦(1<<ADATE) ¦ (1<<ADPS2) ¦ (1<<ADPS1) STS ADCSRA, R16 LDI R16, (1<<ADTS2) ¦ (0<<ADTS1) ¦ (1<<ADTS0) ; Timer1 Compare Match B trigger STS ADCSRB, R16 LDI R16, 0x01 ; Disable digital input on ADC1-ADC7 STS DIDR0, R16 AGAIN: // Wait for the ADIF flag in ADCSRA to become 1, indicating that the conversion is complete. // Once conversion is complete, clear the ADIF flag and send data via UART as per the required format: 0x55 ADCH ADCL 0xFF. //(Todo: Write UART data handling code) LDS R17,ADCSRA SBRS R17,ADIF ; wait for the conversion complete RJMP AGAIN LDS R16, ADCSRA ORI R16, (1<<ADIF) ; Clear ADC interrupt flag STS ADCSRA, R16 RCALL DISPLAY_V IN R17,TIFR1 ;??c c? OCF1A OUT TIFR1,R17 ;xóa c? OCF1A n?u =1 RJMP AGAIN // Subroutines for UART communication ;init UART 0 ;CPU clock is 8Mhz USART_Init: ; Set baud rate to 9600 bps with 8 MHz clock ldi r16, 51 sts UBRR0L, r16 ;set double speed ldi r16, (0 << U2X0) sts UCSR0A, r16 ; Set frame format: 8 data bits, no parity, 1 stop bit ldi r16, (1 << UCSZ01) ¦ (1 << UCSZ00) sts UCSR0C, r16 ; Enable transmitter and receiver ldi r16, (1 << RXEN0) ¦ (1 << TXEN0) ; cho phep thu va phat sts UCSR0B, r16 ret ;send out 1 byte in r16 USART_SendChar: push r17 ; Wait for the transmitter to be ready USART_SendChar_Wait: lds r17, UCSR0A sbrs r17, UDRE0 ;check USART Data Register Empty bit rjmp USART_SendChar_Wait sts UDR0, r16 ;send out pop r17 ret ;receive 1 byte in r16 USART_ReceiveChar: push r17 ; Wait for the transmitter to be ready USART_ReceiveChar_Wait: lds r17, UCSR0A sbrs r17, RXC0 ;check USART Receive Complete bit rjmp USART_ReceiveChar_Wait lds r16, UDR0 ;get data pop r17 ret ; ---------------- TIMER1 Initialization ---------------- TIMER1_INIT: ; Configure Timer1 in CTC Mode (Mode 4) LDI R17, 0x00 STS TCCR1A, R17 ; Normal port operation, no waveform generation ; Load Top Value (OCR1A) LDI R17, HIGH(TOP_VALUE) ; Load high byte of OCR1A STS OCR1AH, R17 LDI R17, LOW(TOP_VALUE) ; Load low byte of OCR1A STS OCR1AL, R17 ; Load Compare Match B value (OCR1B) LDI R17, HIGH(COMPB_VALUE) ; Load high byte of OCR1B STS OCR1BH, R17 LDI R17, LOW(COMPB_VALUE) ; Load low byte of OCR1B STS OCR1BL, R17 ; Start Timer1 with Prescaler = 256 (CS12 = 1, CS11 = 0, CS10 = 0) LDI R17, (1<<WGM12)¦(1<<CS12) STS TCCR1B, R17 ; Enable Compare Match B Interrupt LDI R17, (1<<OCIE1B) STS TIMSK1, R17 SEI ; Enable global interrupts RET ; ---------------- Timer1 Compare Match B ISR ---------------- TIMER1_COMPB_ISR: LDI R17, 0 STS TCNT1H, R17 STS TCNT1L, R17 ; RESET COUNTER RETI ; Return from interrupt ;------------- ;Display DISPLAY_V: ;A*Vref = A*5 LDS AL,ADCL LDS AH,ADCH LDI BL,LOW(5) ;Load multiplier into BH:BL LDI BH,HIGH(5) ; RCALL MUL16x16 ;Reusult is stored in: R21,R20 ;V=x.-------------------------------------------------------------------------------- ;A*5/1024 ;Ketqua: R0:is the integer part, display it on LCD. R2,R3 is the remainder. MOV AL,R20 MOV AH,R21 LDI BL,LOW(1024) ;Load multiplier into BH:BL LDI BH,HIGH(1024) ; RCALL DIV1616 ;display the integer part x._ LDI R17,0xC0 RCALL WRITE_COMMAND MOV R16,R0 LDI R17,0x30 ADD R16,R17 RCALL USART_SendChar MOV R17,R16 RCALL WRITE_DATA LDI R16,46 ; "." RCALL USART_SendChar LDI R17,46 RCALL WRITE_DATA ;V=x.x------------------------------------------------------------------------------- ;remainder*10 MOV AL,R2 MOV AH,R3 LDI BL,LOW(10) ;Load multiplier into BH:BL LDI BH,HIGH(10) ; RCALL MUL16x16 ;Reusult is stored in: R21,R20 ;reamainder*10/1024 ;R0:integer part, display it on LCD. R2,R3 is remainder. MOV AL,R20 MOV AH,R21 LDI BL,LOW(1024) ;Load multiplier into BH:BL LDI BH,HIGH(1024) ; RCALL DIV1616 ;Display integer part _.x MOV R16,R0 LDI R17,0x30 ADD R16,R17 RCALL USART_SendChar MOV R17,R16 RCALL WRITE_DATA ;V=x.xx----------------------------------------------------------------------------- ;So du*10 MOV AL,R2 MOV AH,R3 LDI BL,LOW(10) ;Load multiplier into BH:BL LDI BH,HIGH(10) ; RCALL MUL16x16 MOV AL,R20 MOV AH,R21 LDI BL,LOW(1024) ;Load multiplier into BH:BL LDI BH,HIGH(1024) ; RCALL DIV1616 MOV R16,R0 LDI R17,0x30 ADD R16,R17 RCALL USART_SendChar MOV R17,R16 RCALL WRITE_DATA ;V=x.xxx--------------------------------------------------------------------------------- MOV AL,R2 MOV AH,R3 LDI BL,LOW(10) ;Load multiplier into BH:BL LDI BH,HIGH(10) RCALL MUL16x16 MOV AL,R20 MOV AH,R21 LDI BL,LOW(1024) ;Load multiplier into BH:BL LDI BH,HIGH(1024) ; RCALL DIV1616 MOV R16,R0 LDI R17,0x30 ADD R16,R17 RCALL USART_SendChar MOV R17,R16 RCALL WRITE_DATA LDI R16,0x0A RCALL USART_SendChar LDI R16,0x0D RCALL USART_SendChar MOV R17,R16 RCALL WRITE_DATA RET ;----------------------------------------------------------------------------- .DEF ZERO = R5 ;To hold Zero /*.DEF AL = R16 ;To hold multiplicand .DEF AH = R17 .DEF BL = R18 ;To hold multiplier .DEF BH = R19*/ .DEF ANS1 = R20 ;To hold 32 bit answer .DEF ANS2 = R21 .DEF ANS3 = R22 .DEF ANS4 = R23 LDI AL,LOW(42) ;Load multiplicand into AH:AL LDI AH,HIGH(42) ; LDI BL,LOW(10) ;Load multiplier into BH:BL LDI BH,HIGH(10) ; MUL16x16: CLR ZERO ;Set R2 to zero MUL AH,BH ;Multiply high bytes AHxBH MOVW ANS4:ANS3,R1:R0 ;Move two-byte result into answer MUL AL,BL ;Multiply low bytes ALxBL MOVW ANS2:ANS1,R1:R0 ;Move two-byte result into answer MUL AH,BL ;Multiply AHxBL ADD ANS2,R0 ;Add result to answer ADC ANS3,R1 ; ADC ANS4,ZERO ;Add the Carry Bit MUL BH,AL ;Multiply BHxAL ADD ANS2,R0 ;Add result to answer ADC ANS3,R1 ; ADC ANS4,ZERO ;Add the Carry Bit RET ;----------------------------------------------------------------------------- .DEF ANSL = R0 ;To hold low-byte of answer .DEF ANSH = R1 ;To hold high-byte of answer .DEF REML = R2 ;To hold low-byte of remainder .DEF REMH = R3 ;To hold high-byte of remainder .DEF AL = R16 ;To hold low-byte of dividend .DEF AH = R17 ;To hold high-byte of dividend .DEF BL = R18 ;To hold low-byte of divisor .DEF BH = R19 ;To hold high-byte of divisor .DEF C = R25 ;Bit Counter LDI AL,LOW(420) ;Load low-byte of dividend into AL LDI AH,HIGH(420) ;Load HIGH-byte of dividend into AH LDI BL,LOW(10) ;Load low-byte of divisor into BL LDI BH,HIGH(10) ;Load high-byte of divisor into BH DIV1616: MOVW ANSH:ANSL,AH:AL ;Copy dividend into answer LDI C,17 ;Load bit counter SUB REML,REML ;Clear Remainder and Carry CLR REMH ; LOOP: ROL ANSL ;Shift the answer to the left ROL ANSH ; DEC C ;Decrement Counter BREQ DONE ;Exit if sixteen bits done ROL REML ;Shift remainder to the left ROL REMH ; SUB REML,BL ;Try to subtract divisor from remainder SBC REMH,BH BRCC SKIP ;If the result was negative then ADD REML,BL ;reverse the subtraction to try again ADC REMH,BH ; CLC ;Clear Carry Flag so zero shifted into A RJMP LOOP ;Loop Back SKIP: SEC ;Set Carry Flag to be shifted into A RJMP LOOP DONE:RET // Subroutine ;------------------------------------------------ // POWERUP_LCD_4BIT // Input: None // Output: None // Description: Subroutine to power up the LCD POWERUP_LCD_4BIT: LDI R16, 250 ; Load 250 into R16 RCALL DELAY_US ; Call the delay subroutine LDI R16, 250 ; Load 250 into R16 RCALL DELAY_US ; Call the delay subroutine LDI R17, $30 ; Load command to initialize LCD in 8-bit mode RCALL OUT_COMMAND ; Call to output command LDI R16, 50 ; Load 50 into R16 RCALL DELAY_US ; Call the delay subroutine LDI R17, $30 ; Load command to initialize LCD in 8-bit mode again RCALL OUT_COMMAND ; Call to output command LDI R16, 2 ; Load delay RCALL DELAY_US ; Call the delay subroutine LDI R17, $20 ; Load command for 4-bit mode RCALL OUT_COMMAND ; Call to output command RET // LCD Initialization Subroutine // Input: None // Output: None // Description: Initializes the LCD INIT_LCD_4BIT: LDI R17, $28 ; Command for 4-bit mode, 2 lines, 5x8 dots CALL WRITE_COMMAND ; Call to write command LDI R17, $01 ; Command to clear the display CALL WRITE_COMMAND ; Call to write command LDI R17, $0C ; Command to turn on display, cursor off CALL WRITE_COMMAND ; Call to write command LDI R17, $06 ; Command to shift cursor right CALL WRITE_COMMAND ; Call to write command RET // Subroutine to write command to LCD // Input: R17 // Output: None // Description: Writes command contained in R17 to the LCD. WRITE_COMMAND: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output the first 4 bits RCALL OUT_COMMAND ; Call to output command POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output the last 4 bits RCALL OUT_COMMAND ; Call to output command RET // Subroutine to write data to LCD // Input: R17 // Output: None // Description: Writes character to LCD according to ASCII code; for example, R17=0x41 displays the letter A WRITE_DATA: PUSH R17 ; Save R17 on the stack ANDI R17, $F0 ; Output first 4 bits RCALL OUT_DATA ; Call to output data POP R17 ; Restore R17 from the stack SWAP R17 ; Swap nibbles ANDI R17, $F0 ; Output last 4 bits RCALL OUT_DATA ; Call to output data RET OUT_COMMAND: OUT LCD_PORT, R17 ; Output command to the LCD port CBI LCD_PORT, RS ; Clear RS CBI LCD_PORT, RW ; Clear RW SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET OUT_DATA: OUT LCD_PORT, R17 ; Output data to the LCD port SBI LCD_PORT, RS ; Set RS for data CBI LCD_PORT, RW ; Clear RW SBI LCD_PORT, EN ; Set Enable NOP ; No operation CBI LCD_PORT, EN ; Clear Enable LDI R16, 20 ; Load delay CALL DELAY_US ; Call delay subroutine RET /* DELAY_US Input: R16 Output: None Description: DELAY R16*100 MICROSEC */ DELAY_US: MOV R15, R16 ; Move R16 to R15 LDI R16, 200 ; Load delay value L1: MOV R14, R16 ; Move delay value to R14 L2: DEC R14 ; Decrement R14 NOP ; No operation BRNE L2 ; Branch if not equal to zero DEC R15 ; Decrement R15 BRNE L1 ; Branch if not equal to zero RET LINE1: .DB "the value is ",$00 ; Define string for line 1