# OBJECTIVE:

- Implement I/O Port interface and calculation instructions.

# REFERENCES:

- Lab Manual Chapter 1-2

# EXPERIMENT 1:

- Connect one AVR port (e.g., PORT A) to a dip switch. Connect another port to a LED bar (e.g., PORT B).

- Write a program to continuously read the state of the dip switch and send it to the LED. If the switch is in the OFF state, the corresponding LED will turn off.

# EXPERIMENT 2:

- Write a program to read the value of the port connected to the dip switch, add 5 to it, and send it to the port connected to the LED bar.

- Change the state of the dip switch and observe the status of the LED bar.

# EXPERIMENT 3:

- Connect and implement a program to calculate the product of two nibbles (high and low) of PORT A and send it to PORT B. Consider these two nibbles as unsigned numbers.

Example: PORT A = 0b0111_1111, then PORT B = 3 * 15.

- Change the state of the dip switch and observe the status of the LED bar.

# EXPERIMENT 4:

- Connect and implement a program to calculate the product of two nibbles (high and low) of PORT A and send it to PORT B. Consider these two nibbles as signed numbers.

Example: PORT A = 0b0111_1111, then PORT B = 3 * (-1).

- Change the state of the dip switch and observe the status of the LED bar.

# EXPERIMENT 5:

- Connect PA0 to a single switch and PA1 to a single LED on the LED block (note that they are from the same port).

- Write a program to turn on the LED if the switch is pressed and turn it off if the switch is released.

# EXPERIMENT 1

- Answer the following questions:

  - How do you retrieve values from the two nibbles of PORT A?

- Use the “IN” instruction to copy those values into a register.

  - How do you enable pull-up resistors?

- Set PORTX to high by add 0xFF to them.

  - When the switch is in the ON/OFF state, what is the pin value of the port?

- If the pull-up resistors are enabled, pin will get 1 at OFF state and 0 at ON state (of the switch).

  - When the port pin is in state 1, is the BAR LED on or off?

- Pin is in state 1 so BAR LED is on.

  - Source code with comments.

START: LDI R17, $00 ;LOAD R17 = 00 LDI R16, $FF ;LOAD R16 = FF OUT DDRB, R16 ;SET DDRD = 1 --> OUTPUT OUT DDRA, R17 ;SET DDRB = 0 --> INPUT OUT PORTA, R16 ;SET PULL UP REGISTER AT PORTA READ_PIN: IN R18, PINA ;READ INPUT VALUE FROM PINA TO R18 OUT PORTB, R18 ;LOAD VALUE OF R18 TO PORTB RJMP READ_PIN ;READ AGAIN

# EXPERIMENT 2

- Answer the following questions:

  - Source code with comments.

START: LDI R19, $05 ;LOAD R19 = 5 LDI R17, $00 ;LOAD R17 = 00 LDI R16, $FF ;LOAD R16 = FF OUT DDRB, R16 ;SET DDRD = 1 --> OUTPUT ; OUT DDRA, R17 ;SET DDRB = 0 --> INPUT OUT PORTA, R16 ;SET PULL UP REGISTER AT PORTA READ_PIN: IN R18, PINA ;READ INPUT VALUE FROM PINA TO R18 LDI R20, 0x05 ;LOAD 5 TO R20 ADD R18, R20 ;ADD 5 TO R18 OUT PORTB, R18 ;LOAD VALUE OF R18 TO PORTB RJMP READ_PIN ;READ AGAIN

# EXPERIMENT 3

- Answer the following questions:

  - How do you retrieve values from the two nibbles of PORT A?

- First, we get low nibble by doing logic AND 0F with value in PORTA and store them to a register. Second, use SWAP to change the high nibble back and do logic AND 0F with the swapped value and store to another register. Now we have 2 nibbles.

  - Source code with comments.

START: LDI R17, $00 ;LOAD R16 = 00 LDI R16, $FF ;LOAD R16 = FF OUT DDRB, R16 ;SET DDRB = 1 --> OUTPUT ; OUT DDRA, R17 ;SET DDRA = 0 --> INPUT OUT PORTA, R16 ;SET PULL UP REGISTER AT PORTA READ_PIN: IN R18, PINA ;READ INPUT VALUE FROM PINA TO R18 ANDI R18, $0F ;AND R18 WITH 00001111 TO GET LOW NIBBLE IN R19, PINA ;READ INPUT VALUE FROM PINA TO R19 SWAP R19 ;SWAP NIBBLES TO CHANGE THE HIGH NIBBLE TO LOW NIBBLE ANDI R19, $0F ;AND R19 WITH 00001111 TO GET THE HIGH NIBBLE AS WE DID THE SWAP CALL MULTIPLY ;DO MULTIPLICATION RJMP READ_PIN ;TURN BACK TO CHECK INPUT MULTIPLY: MUL R18, R19 ;DO MULTIPLICATION, RESULT WILL BE STORE IN R0 OUT PORTB, R0 ;LOAD THE RESULT TO PORTB, USE R0 AS THIS STORE THE LOW BITS RET

# EXPERIMENT 4

- Answer the following questions:

  - Source code with comments.

START: LDI R16, $FF ;LOAD R16 = FF OUT DDRB, R16 ;SET DDRB = 1 --> OUTPUT ; LDI R17, $00 ;LOAD R17 = 00 OUT DDRA, R17 ;SET DDRA = 0 --> INPUT ; OUT PORTA, R16 ;SET PULL UP REGISTER AT PORTA READ_PIN: IN R18, PINA ;READ INPUT VALUE FROM PINA TO R18 ANDI R18, $F0 ;AND R18 WITH 00001111 TO GET HIGH NIBBLE IN R19, PINA ;READ INPUT VALUE FROM PINA TO R19 SWAP R19 ;SWAP NIBBLES TO CHANGE THE HIGH NIBBLE TO LOW NIBBLE ANDI R19, $F0 ;AND R19 WITH 00001111 TO GET THE LOW NIBBLE AS WE DID THE SWAP CALL MULTIPLY ;DO MULTIPLICATION RJMP READ_PIN ;TURN BACK TO CHECK INPUT MULTIPLY: MULS R18, R19 ;DO MULTIPLICATION, RESULT WILL BE STORE IN R0 OUT PORTB, R1 ;LOAD THE RESULT TO PORTB, USE R1 AS THIS STORE THE HIGH BITS RET

# EXPERIMENT 5

- Answer the following questions:

  - When the switch is pressed/released, what is the pin value of the port?

- In order to get the kit work properly, we need to set pull-up resistor at PA0. As a result, when we press the button, PA0’s value is 0 and when we release, it will get 1. So with 1 we turn off the LED and with 0 we turn it on.

  - To make the LED light up, what logic level should the port pin output?

- LED light up so the pin PA1 need to be 1 but PA0 is 0 and vice versa.

  - Source code with comments.

START: LDI R16, 0x02 ; Set pin 1 of PORTA as output OUT DDRA, R16 ; Configure DDRA: pin 1 as output (0b00000010), pin 0 remains input SBI PORTA, PA0 ; SET PULL UP REGISTER CHECK: IN R17, PINA ; Read from PINA (get the input value from pin 0) ANDI R17, 0x01 CPI R17, 0x01 BREQ TURN_OFF BRNE TURN_ON RJMP CHECK ; Keep checking continuously TURN_ON: SBI PORTA, PA1 RJMP CHECK TURN_OFF: CBI PORTA, PA1 RJMP CHECK