# objective:

- Execute instructions to create delays using subroutines.

- Perform communication with shift registers.

# references:

- Lab manual chapter 1-2.

# EXPERIMENT 1:

Use the following program:

.include "m324PAdef.inc" .org 00 ldi r16,0x01 out DDRA, r16 start: sbi PORTA,PINA0 cbi PORTA, PINA0 - rjmp start

- Connect PA0 to a measurement channel on the TEST STATION and measure pulse forms using an oscilloscope.

# EXPERIMENT 2:

- Write a subroutine Delay1ms and use it to write a program to generate a 1KHz square wave on PA0.

- Use this subroutine to write subroutines Delay10ms, Delay100ms, and Delay1s.

- Use the Delay1s subroutine to write a program to blink/turn off an LED connected to PA0.

# EXPERIMENT 3:

- Connect the necessary signals from an AVR port to the control signals of the shift register on header J13. Connect the output of the shift register to a LED bar.

- Using the sample programs from the experiment guide, write a program to create a gradually lit LED effect from left to right, then gradually turn them off from left to right after every 500ms.

# EXPERIMENT 1:

- Answer the following questions:

  - Capture a pulse waveform on PA0.

  - What is the frequency, duration of the high signal, and duration of the low signal?

  - Explain the measured results.

- a/

# EXPERIMENT 2:

- Answer the following questions:

  - How to calculate the number of machine cycles needed to execute the Delay1ms subroutine. Present a simulation image.

  - Image of a 1KHz square wave on PA0.

  - What is the error?

- Source code for 2.c with comments.

- Delay1ms: - /*XTAL is 8MHz, we scale down 64*/ - LDI R16,(1<<0) ;SET BIT 0 - SBI DDRA,0 - LDI R17,0 - OUT PORTA,R17 - LDI R20,124 ;STORE 124 INTO OCR0A - OUT OCR0A,R20 - Delay1ms: - LDI R20,0x02 ;SET WGM01 ON - OUT TCCR0A,R20 - LDI R20,0x03 ;SET CSO1 AND CSO2 - OUT TCCR0B,R20 - AGAIN: - SBIS TIFR0,OCF0A - RJMP AGAIN - LDI R20,0x0 - OUT TCCR0B,R20 - LDI R20,1<<OCF0A - OUT TIFR0,R20 - EOR R17,R16 - OUT PORTA,R17 - RJMP Delay1ms - Delay10ms: - /*XTAL is 8MHz, we scale down 64*/ - LDI R18, 10 ;SET R18 =10 - LDI R16,(1<<0) ;SET BIT 0 - SBI DDRA,0 - LDI R17,0 - OUT PORTA,R17 - LDI R20,124 ;STORE 124 INTO OCR0A - OUT OCR0A,R20 - Delay10ms: - CALL Delay1ms ;CALL DELAY 1ms - DEC R18 ;AFTER FINISH DELAY1ms, REDUCE R18 - BRNE Delay10ms ;IF FINISH 10 TIMES, SKIP THIS - EOR R17,R16 ;CHANGE OUTPUT VALUE - OUT PORTA,R17 ;OUT - LDI R18, 10 ;RESET R18 =10 - RJMP Delay10ms ;REPEAT - Delay1ms: - LDI R20,0x02 ;SET WGM01 ON - OUT TCCR0A,R20 - LDI R20,0x03 ;SET CSO1 AND CSO2 - OUT TCCR0B,R20 - AGAIN: - SBIS TIFR0,OCF0A - RJMP AGAIN - LDI R20,0x0 - OUT TCCR0B,R20 - LDI R20,1<<OCF0A - OUT TIFR0,R20 - RET - Delay100ms: - /*XTAL is 8MHz, we scale down 64*/ - LDI R18, 100 ;SET R18 =100 - LDI R16,(1<<0) ;SET BIT 0 - SBI DDRA,0 - LDI R17,0 - OUT PORTA,R17 - LDI R20,124 ;STORE 124 INTO OCR0A - OUT OCR0A,R20 - Delay100ms: - CALL Delay1ms ;CALL DELAY 1ms - DEC R18 ;AFTER FINISH DELAY1ms, REDUCE R18 - BRNE Delay100ms ;IF FINISH 10 TIMES, SKIP THIS - EOR R17,R16 ;CHANGE OUTPUT VALUE - OUT PORTA,R17 ;OUT - LDI R18, 100 ;RESET R18 =100 - RJMP Delay100ms ;REPEAT - Delay1ms: - LDI R20,0x02 ;SET WGM01 ON - OUT TCCR0A,R20 - LDI R20,0x03 ;SET CSO1 AND CSO2 - OUT TCCR0B,R20 - AGAIN: - SBIS TIFR0,OCF0A - RJMP AGAIN - LDI R20,0x0 - OUT TCCR0B,R20 - LDI R20,1<<OCF0A - OUT TIFR0,R20 - RET - Delay1s: - /*XTAL is 8MHz, we scale down 64*/ - LDI R18, 100 ;SET R18 =100 - LDI R19, 10 ;SET R19 =10 - LDI R16,(1<<0) ;SET BIT 0 - SBI DDRA,0 - LDI R17,0 - OUT PORTA,R17 - LDI R20,124 ;STORE 124 INTO OCR0A - OUT OCR0A,R20 - Delay1s: - CALL Delay100ms - DEC R19 - BRNE Delay1s - EOR R17,R16 ;CHANGE OUTPUT VALUE - OUT PORTA,R17 ;OUT - LDI R19, 10 ;RESET R19 =10 - RJMP Delay1s ;REPEAT - Delay100ms: - CALL Delay1ms ;CALL DELAY 1ms - DEC R18 ;AFTER FINISH DELAY1ms, REDUCE R18 - BRNE Delay100ms ;IF FINISH 100 TIMES, SKIP THIS - LDI R18, 100 ;RESET R18 =100 - RET - Delay1ms: - LDI R20,0x02 ;SET WGM01 ON - OUT TCCR0A,R20 - LDI R20,0x03 ;SET CSO1 AND CSO2 - OUT TCCR0B,R20 - AGAIN: - SBIS TIFR0,OCF0A - RJMP AGAIN - LDI R20,0x0 - OUT TCCR0B,R20 - LDI R20,1<<OCF0A - OUT TIFR0,R20 - RET

# EXPERIMENT 3:

- Answer the following questions:

  - Describe the connections on the experimental kit.

  - According to the datasheet of 74HC595, what is the highest clock frequency it can operate at?

  - How do you expand the display to 16 LEDs?

- Source code with comments.